Massachusetts Institute of Technology

16.831 — Space Systems Engineering

Radios Lab — Spring 2026

Part 3: Packets, Commands & Image Transfer — Student Study Guide

# Part 12 — Simple Commands (Warm-up)

Before tackling image transfer, let’s get the Pi-to-Pi link doing something useful: sending typed commands from the ground station and getting real data back.

## 12.1  The Command/Response Pattern

How satellite commanding works Every satellite has a command dictionary — a list of numbered commands the ground station can send. The satellite receives a command byte, looks it up, executes the right action, and sends back a response. Think of it like a TV remote: pressing button 5 always changes to channel 5, pressing 7 always changes to channel 7. In this lab, byte 0 of every 32-byte payload is the command type. The server reads byte 0 first and decides what to do. All remaining bytes are context (parameters, data, or zeros).

## 12.2  New Imports

Add these two imports at the top of BOTH Python files, directly below the existing imports:

import os # file system operations (needed for image transfer later) import struct # packing/unpacking numbers into bytes

## 12.3  New Constants to Add

Add the following constant block to BOTH Python files. The values must be identical on both sides.

All 32 bytes are yours Unlike previous labs, the full 32-byte SMBUS payload is available to Python. The Arduino handles the RF checksum completely transparently — it appends the checksum byte to the 36-byte RF packet before transmitting, and strips it off on receive. Your Python code never sees it.

| Constant name | Value | Meaning |
| --- | --- | --- |
| SMBUS_MAX_BYTES | 32 | Max bytes per I2C transfer — fully usable by Python |
| CHUNK_DATA_SIZE | 27 | Image data bytes per chunk packet (bytes 5–31) |
| TIMEOUT_S | 2.0 | Seconds to wait for a reply before giving up |
| CMD_TELEM_A | 0x01 | Request/response: telemetry set A |
| CMD_TELEM_B | 0x02 | Request/response: telemetry set B |
| CMD_IMG_REQUEST | 0x10 | Client → server: “I want this image” |
| CMD_IMG_CONFIRM | 0x11 | Server → client: image found, here is metadata |
| CMD_IMG_NACK | 0x13 | Server → client: image not found |
| CMD_IMG_ACK | 0x12 | Client → server: acknowledged, start transfer |
| CMD_CHUNK_REQ | 0x14 | Client → server: send me chunk number N |
| CMD_CHUNK_DATA | 0x15 | Server → client: here is chunk number N |
| CMD_IMG_DONE | 0x16 | Client → server: all chunks received, transfer done |

## 12.4  Two New Helper Functions (both files)

Add these two helper functions to BOTH Python files, near the existing readDataI2C() function.

### sendData(data)

- Takes a bytearray or list of up to 32 bytes.

- Creates a 32-byte bytearray filled with zeros.

- Copies your data in, then calls the I2C write function.

### waitForMessage(timeout=TIMEOUT_S)

- Polls the GPIO pin in a tight loop.

- Records the start time so it can detect a timeout.

- Returns True as soon as the pin goes HIGH (message ready).

- Returns False if the elapsed time exceeds the timeout.

- This replaces the bare while-loop in the original scripts.

## 12.5  Modifying the Server Main Loop

Replace the existing while True loop in the server. The new structure is a simple state machine: wait for a message, read the command byte, and dispatch to the right handler.

- Wait for a message using your new waitForMessage() function.

- Read 32 bytes over I2C.

- Read byte 0 as the command type.

- Use if/elif/else to handle each command type:

  - If CMD_TELEM_A: call your telemetry A builder and send the result.

  - If CMD_TELEM_B: call your telemetry B builder and send the result.

  - Else: print an unknown command warning.

### buildTelemetryA() — server only

- Creates a 32-byte bytearray.

- Sets byte 0 to CMD_TELEM_A.

- Packs three spoofed floats into bytes 1–12 using the struct module (4 bytes each):

  - Temperature (°C) at bytes 1–4

  - Battery voltage (V) at bytes 5–8

  - Altitude (km) at bytes 9–12

- Returns the bytearray.

struct format codes quick reference '>f' = big-endian 4-byte float e.g. 23.5 '>H' = big-endian 2-byte uint16 e.g. 2000 '>I' = big-endian 4-byte uint32 e.g. 83886 The '>' prefix means big-endian (most significant byte first) — always use this for consistency across Pi and Arduino.

### buildTelemetryB() — server only

Same structure as Telemetry A but pack roll, pitch, and yaw angles instead. Choose your own spoofed values.

## 12.6  Modifying the Client

Replace the one-shot client script with an interactive loop:

- Print a simple command menu: t = Telemetry A, u = Telemetry B, i = Image, q = Quit.

- Use input() to read a keypress.

- For t and u: build a 32-byte packet with the command byte set, send it, wait for a response, parse and print the three floats.

- For parsing: use struct to read each float from the response at the correct byte offset.

# Part 13 — Packet Theory

## 13.1  What Does the Complete RF Packet Look Like?

When the Arduino transmits, it doesn’t just send your 32-byte payload. It wraps it in a header and appends a checksum. Here is the complete structure of every RF packet on the wire:

Complete RF Packet transmitted by Arduino over 915 MHz Total: 36 bytes Byte(s) Size Field Written by Description ────────────────────────────────────────────────────────────────────── 0 1 B PREAMBLE Arduino Magic byte 0xAA: marks a valid packet 1 1 B DEST_ADDR Arduino Who this packet is addressed to 2 1 B SRC_ADDR Arduino Who is sending this packet 3 – 34 32 B PAYLOAD Python (Pi) Your data — all 32 bytes, fully usable 35 1 B CHECKSUM Arduino XOR of bytes 0–34; appended before TX ────────────────────────────────────────────────────────────────────── Receive side (Arduino): XOR all 36 bytes → result must equal 0x00 If result != 0x00 → BAD CHECKSUM, discard, do NOT signal Pi If result == 0x00 → strip header (bytes 0-2) and checksum (byte 35), pass PAYLOAD (bytes 3-34) to Pi via I2C The Python scripts on the Pi see ONLY the 32-byte PAYLOAD. The header and checksum are invisible to Python — handled entirely by Arduino.

Key point: Python is always working with the 32-byte PAYLOAD only The 3-byte header (preamble, addresses) was added in Part 10. The 1-byte checksum is added automatically by the Arduino transmit-side code from Part 14. Your Python scripts only ever write or read the 32-byte PAYLOAD. The wrapping and unwrapping is completely handled by the Arduinos.

## 13.2  The Problem with Raw Bytes

Right now, if a bit flips in transit and the receiver gets corrupted data, it has no idea. There’s also no way to tell if a packet arrived out of order or never arrived at all. Fixing these two problems requires checksums and sequence numbers.

## 13.3  Checksums

What is a checksum? Imagine you’re reading a sentence over a noisy phone. To check the other person heard correctly, you say: “read back every third word.” A checksum is the mathematical version of that. The sender computes a short fingerprint of the data and sends it along. The receiver recomputes the fingerprint from the data it received — if the fingerprints match, the data is intact.

### XOR checksum — the simplest kind

XOR every byte in the packet together into a single byte. Append that byte. On receive, XOR all bytes including the checksum — the result will be 0 if nothing was corrupted.

Example: data = [0x12, 0x34, 0x56] 0x12 = 0001 0010 0x34 = 0011 0100 XOR = 0010 0110 = 0x26 0x26 = 0010 0110 0x56 = 0101 0110 XOR = 0111 0000 = 0x70 <— checksum, appended to packet Receiver gets [0x12, 0x34, 0x56, 0x70]: XOR all four → 0x70 XOR 0x70 = 0x00 ✓ GOOD If 0x34 was corrupted to 0x35 in transit: XOR result ≠ 0x00 ✗ BAD — discard packet

- XOR checksums catch single-bit errors reliably.

- They are not foolproof — some multi-bit errors cancel out. Real systems use CRC-16 or CRC-32, but XOR is fine for this class.

Bad packets vs. lost packets — they’re not the same thing in real life In this class, bad packets and lost packets are treated identically: the client simply re-requests them. In a real satellite system, though, they tell you different things: Lost packet: you heard nothing. The packet may not have been transmitted, or it was absorbed by the environment before reaching you. Bad packet (failed checksum): you heard something, but it was corrupted. This is actually useful! It tells you the radio link EXISTS — the satellite is transmitting and within range — but RF conditions are poor (interference, obstruction, low signal strength). A ground station that sees a rising bad-packet rate can respond by increasing transmit power, switching to a more robust modulation, or repositioning the antenna. We simplify to keep the code manageable. Just be aware the distinction matters in a real system.

## 13.4  Sequence Numbers

Without sequence numbers, if the server sends chunks 0, 1, 2, 3 and the client only receives 0, 1, 3 — it has no idea chunk 2 is missing. It just sees four packets and assumes it has everything.

- Every chunk packet carries its own sequence number (like page numbers in a book).

- The client tracks which numbers it has and which it is still waiting for.

- Any gaps in the received sequence = re-request those numbers.

## 13.4  Lost Packet Recovery: Two Strategies

| Strategy | Full name | How it works | Best for |
| --- | --- | --- | --- |
| ARQ | Automatic Repeat reQuest | Request each packet. If no reply within timeout → immediately re-request before moving on. | Low-latency links, small files |
| Bulk re-request | — | Request all packets in one pass. At the end, re-request all missing ones. Repeat until complete. | Noisy links, large files |

You will implement bulk re-request. The key data structure is a dictionary mapping chunk number to received bytes. Start with a list of all expected chunk numbers; anything not in the dictionary after one pass is re-requested.

## 13.5  A Note on Space Systems — Why Real Satellites Use a Different Protocol

The speed-of-light problem The systems above assume you can send a request and get a reply in under a second. In space, distances create unavoidable delays even at the speed of light: Moon: ∿ 1.3 seconds one-way Mars (close): ∿ 3 minutes one-way Saturn: ∿ 80 minutes one-way Waiting 160 minutes for a single reply is not practical. Real satellites use a Store-and-Forward approach instead: 1. The satellite dumps ALL stored data continuously during a ground pass. 2. The ground station records everything and notes which packet numbers are missing. 3. On the NEXT ground pass, the ground station sends a list of all missing packets. The satellite retransmits only those. 4. This repeats until the ground station has everything. Crucially, this is NOT a server-client relationship. There is no real-time back-and-forth. Each ground pass is a one-way burst. The protocol real satellites use for this is called CCSDS (Consultative Committee for Space Data Systems).

# Part 14 — Checksums in Arduino

Unlike other sections, you are given the complete Arduino code for this part. Add it to BOTH .ino sketches exactly as described.

## 14.1  How the Checksum Fits In

Arduino appends the checksum, Python never sees it The Pi sends a 32-byte payload. The Arduino takes that 32-byte payload, prepends the 3-byte address header (from Part 10), then XORs all 35 bytes together and appends the result as a 36th byte. The full 36-byte packet is what goes over RF. The receiving Arduino XORs all 36 received bytes. If the result is 0x00, the data is clean. It then strips the header (bytes 0-2) and checksum (byte 35), and passes the original 32-byte payload to the Pi via I2C. The Python scripts see nothing but their 32-byte payload.

## 14.2  Transmit Side (Both Sketches)

// ── XOR checksum: compute over all 35 bytes, append as byte 35 ───────── // pktBuf currently holds: [preamble][dest][src][32 payload bytes] = 35 bytes // We expand pktBuf by 1 byte in the transmit path: uint8_t txBuf[sizeof(pktBuf) + 1]; memcpy(txBuf, pktBuf, sizeof(pktBuf)); uint8_t chksum = 0; for (int i = 0; i < (int)sizeof(pktBuf); i++) { chksum ^= pktBuf[i]; } txBuf[sizeof(pktBuf)] = chksum; // append checksum as final byte // ─────────────────────────────────────────────────────────────────────

Then change the send line itself to use txBuf:

rf69.send(txBuf, sizeof(txBuf)); // was: rf69.send(pktBuf, sizeof(pktBuf)); rf69.waitPacketSent();

## 14.3  Receive Side (Client & Server are different)

Find the receive block inside loop(). You need to receive into a 36-byte buffer, verify the checksum, and then proceed only if it passes. Replace the existing rf69.recv() call and the address check with:

Client Code:

// ── Receive into a 36-byte buffer (35 header+payload + 1 checksum) ───── uint8_t rxBuf[sizeof(pktBuf) + 1]; uint8_t rxLen = sizeof(rxBuf); if (rf69.recv(rxBuf, &rxLen)) { // Step 1: verify XOR checksum (all 36 bytes XOR'd must equal 0) uint8_t verify = 0; for (int i = 0; i < rxLen; i++) { verify ^= rxBuf[i]; } if (verify != 0) { Serial.println("BAD CHECKSUM — corrupted packet, discarding"); // Treated same as lost packet — do nothing, loop continues } // Step 2: address check (from Part 10) else if (rxBuf[0] != PREAMBLE ¦¦ rxBuf[1] != MY_ADDR) { Serial.println("Not our packet — discarding"); } // Step 3: valid! strip header and checksum, copy payload else { memcpy(dnlinkBuf, rxBuf + HEADER_LEN, SMBUS_MAX_BYTES); GotMsg = true; Serial.println("Valid packet received"); } } // ─────────────────────────────────────────────────────────────────────

Server Code:

// ── Receive into a 36-byte buffer (35 header+payload + 1 checksum) ───── uint8_t rxBuf[sizeof(pktBuf) + 1]; uint8_t rxLen = sizeof(rxBuf); if (rf69.recv(rxBuf, &rxLen)) { // Step 1: verify XOR checksum (all 36 bytes XOR'd must equal 0) uint8_t verify = 0; for (int i = 0; i < rxLen; i++) { verify ^= rxBuf[i]; } if (verify != 0) { Serial.println("BAD CHECKSUM — corrupted packet, discarding"); // Treated same as lost packet — do nothing, loop continues } // Step 2: address check (from Part 10) else if (rxBuf[0] != PREAMBLE ¦¦ rxBuf[1] != MY_ADDR) { Serial.println("Not our packet — discarding"); } // Step 3: valid! strip header and checksum, copy payload into uplinkBuf else { memcpy(uplinkBuf, rxBuf + HEADER_LEN, SMBUS_MAX_BYTES); GotMsg = true; Serial.println("Valid packet received"); } } // ─────────────────────────────────────────────────────────────────────

⚠️ Re-upload both sketches after adding the checksum code The Arduino will not use the new code until it has been uploaded. After uploading, the Serial Monitor should show 'BAD CHECKSUM' for any corrupted packets, and 'Valid packet received' for clean ones.

# Part 15 — Image Transfer Protocol Design

## 15.1  The Core Problem

A typical JPEG photo from a Raspberry Pi camera is 1–2 MB. Your packet payload is 32 bytes, with 27 bytes available for image data per chunk (the rest goes to the command byte and chunk number). That means:

2,000,000 bytes ÷ 27 bytes/chunk = 74,074 chunks uint16 max = 65,535 ✗ NOT enough for a 2 MB image uint32 max = 4,294,967,295 ✓ more than sufficient

- Use uint32 (4 bytes, '>I' in struct) for all chunk numbers in this lab.

- Recommend using small test images (∿5 KB JPEG thumbnails) while developing.

- At 2 seconds per round-trip, a 5 KB image (∿190 chunks) takes about 6 minutes. Reduce TIMEOUT_S once things are working.

## 15.2  Full Packet Layout Reference

Every payload is exactly 32 bytes. The layout of those bytes depends on the command type:

CMD_TELEM_A / B (request from client, or response from server) [0] command byte (0x01 or 0x02) [1–4] float: value 1 (e.g. temperature) [5–8] float: value 2 (e.g. voltage) [9–12] float: value 3 (e.g. altitude) [13–31] zeros CMD_IMG_REQUEST (client → server) [0] 0x10 [1–30] filename as ASCII bytes, null-terminated (max 30 chars) [31] 0 (unused — filename capped at 30 chars) CMD_IMG_CONFIRM (server → client) — image found [0] 0x11 [1] status: 0x01 = found [2–5] total_chunks (uint32, big-endian) [6] bytes in the last chunk (1–27) [7–31] zeros CMD_IMG_NACK (server → client) — image NOT found [0] 0x13 [1–31] zeros CMD_IMG_ACK (client → server) [0] 0x12 [1–31] zeros CMD_CHUNK_REQ (client → server) [0] 0x14 [1–4] chunk_number (uint32, big-endian) [5–31] zeros CMD_CHUNK_DATA (server → client) [0] 0x15 [1–4] chunk_number (uint32, big-endian) [5–31] image data bytes (27 bytes; last chunk may be fewer) CMD_IMG_DONE (client → server) [0] 0x16 [1–31] zeros

## 15.3  Handshake Flow Diagram

Including the NACK path for when the requested file does not exist on the server:

Client (Ground Station) Server (Satellite) ¦ ¦ ¦---[CMD_IMG_REQUEST: 'photo.jpg']-------->¦ ¦ ¦ ¦ File not found:¦ ¦<─────[CMD_IMG_NACK]──────────────── ¦ ¦ Print 'image not found', return False ¦ ¦ ¦ ¦ File found: ¦<─────[CMD_IMG_CONFIRM: 1896 chunks]─── ¦ ¦ status=0x01, total=1896, last_sz=22 ¦ ¦ ¦ ¦---[CMD_IMG_ACK]------------------------->¦ ¦ ¦ ¦---[CMD_CHUNK_REQ: chunk 0]-------------->¦ ¦<───[CMD_CHUNK_DATA: chunk 0, 27 bytes]───¦ ¦ ¦ ¦---[CMD_CHUNK_REQ: chunk 1]-------------->¦ ¦<───[CMD_CHUNK_DATA: chunk 1, 27 bytes]───¦ ¦ ... ¦ ¦---[CMD_CHUNK_REQ: chunk 417]------------>¦ ¦ (no response — packet lost, timeout) ¦ ¦ ... ¦ ¦ (finish first pass, note chunk 417 missing) ¦---[CMD_CHUNK_REQ: chunk 417]------------>¦ <- re-request ¦<───[CMD_CHUNK_DATA: chunk 417]──────── ¦ ¦ ... ¦ ¦---[CMD_IMG_DONE]------------------------>¦ ¦ ¦

# Part 16 — Server-Side Implementation

Extend the server Python script you built in Part 12. Work through the functions below in order.

## 16.1  New Server Constant

- Add a constant that holds the folder path where images are stored on the server Pi.

- Create that folder on RPi B and place a small test JPEG inside it.

## 16.2  buildImgConfirm(filename)

Write this function on the server. Its job is to check whether the requested file exists, calculate how many chunks it will take to transfer, and package that information into a response packet.

Function specification Input: filename (string) — the name of the image file, e.g. 'photo.jpg' Output: a tuple of three values: - packet (bytearray, 32 bytes) — the CMD_IMG_CONFIRM or CMD_IMG_NACK payload, ready to send - total_chunks (int) — number of chunks the image will be split into; 0 if file not found - last_chunk_sz (int) — how many data bytes are in the final chunk (1–27); 0 if file not found

What the function should do, step by step:

- Construct the full file path by combining the image directory constant with the filename argument.

- Check whether the file exists at that path. If it does not, return a CMD_IMG_NACK packet and zeros for the other two return values.

- If the file exists, find out its size in bytes.

- Calculate the total number of chunks needed. Think about what happens if the file divides exactly evenly — is there a remainder? Use ceiling division to handle this correctly.

- Calculate how many bytes go into the final chunk. Watch out for the edge case where the file size is an exact multiple of CHUNK_DATA_SIZE — in that case the last chunk is a full chunk, not zero bytes.

- Build a CMD_IMG_CONFIRM packet, packing total_chunks as a 4-byte big-endian integer into the right bytes, and last_chunk_sz into another byte.

- Return all three values.

Helpful hints Use the os.path module to check if a file exists and to get its size — Python’s documentation for os.path will show you the right functions. The struct module’s pack_into function lets you write a number directly into a specific position inside a bytearray — you tell it the format, the buffer, the offset, and the value. For ceiling division in Python without importing anything extra: (a + b - 1) // b gives you ceiling(a/b) using only integer arithmetic.

## 16.3  getChunkPacket(img_data, chunk_num, total_chunks, last_chunk_sz)

Write this function on the server. Its job is to slice the correct bytes out of the image data for a given chunk number, and package them into a response packet.

Function specification Inputs: - img_data (bytes) — the entire image file loaded into memory as a bytes object - chunk_num (int) — which chunk to serve, zero-indexed - total_chunks (int) — the total number of chunks for this image - last_chunk_sz (int) — how many data bytes are in the final chunk Output: packet (bytearray, 32 bytes) — the CMD_CHUNK_DATA payload, ready to send

What the function should do, step by step:

- Set byte 0 to CMD_CHUNK_DATA.

- Write the chunk number into bytes 1 through 4 as a 4-byte big-endian integer.

- Calculate where in the image data this chunk starts. Every chunk is CHUNK_DATA_SIZE bytes, so the start position is chunk_num multiplied by CHUNK_DATA_SIZE.

- Decide how many bytes to slice. For all chunks except the last, slice exactly CHUNK_DATA_SIZE bytes. For the last chunk, slice only last_chunk_sz bytes.

- Copy the sliced bytes into the packet starting at byte 5.

- Return the 32-byte packet.

Helpful hints You can slice a bytes object just like a list in Python: data[start:end] gives you bytes from index start up to (but not including) index end. The last chunk is chunk number (total_chunks - 1). Be careful: it's the last index, not total_chunks itself. Once you have your slice, you can copy it into the packet using bytearray slice assignment.

## 16.4  Extending the Server Main Loop

Goal: add a new state to the server’s command dispatcher — alongside the existing CMD_TELEM_A and CMD_TELEM_B states — that handles CMD_IMG_REQUEST.

When the server receives CMD_IMG_REQUEST, it should:

- Extract the filename from the incoming 32-byte message. The filename is stored in bytes 1 through 30 as ASCII characters, followed by null bytes (0x00) as padding.

- Convert those raw bytes into a Python string. Hint: first split the bytes at the first null byte to remove padding, then decode the result as ASCII.

- Call buildImgConfirm() with the filename string to check for the file and get the metadata.

- Send the result from buildImgConfirm() — this will be either a CMD_IMG_NACK or a CMD_IMG_CONFIRM packet.

- If the file was not found (total_chunks == 0), print a message and return to the top of the outer loop to wait for the next command.

- If the file was found, wait for CMD_IMG_ACK from the client. If it times out, abort and return to the outer loop.

- Load the entire image file into memory as a bytes object.

- Enter the chunk-serving sub-loop (Section 16.5 below).

Hint: decoding the filename The raw bytes from the message will look like: [0x70, 0x68, 0x6f, 0x74, 0x6f, 0x2e, 0x6a, 0x70, 0x67, 0x00, 0x00, ...] Step 1: get a bytes object from bytes 1 to 30 of the message. Step 2: use the split method on bytes objects to split at b'\x00' (the null byte) — take the first part. Step 3: call .decode('ascii') on that result to get a string. Write each step on a separate line rather than chaining them together.

## 16.5  The Chunk-Serving Sub-Loop

Goal: stay inside a loop serving individual chunk requests until the client signals that it has everything it needs.

Requirements:

- Use a while True loop that only exits when CMD_IMG_DONE is received or when an error occurs.

- At the top of each iteration, call waitForMessage() with a 5-second timeout (longer than the default, since the client may be busy processing). If it times out, the client has disconnected — print a warning and break out of the sub-loop to return to the outer command loop.

- Read the incoming 32-byte message.

- If the command byte is CMD_CHUNK_REQ:

  - Extract the chunk number. It is stored as a 4-byte big-endian integer in bytes 1 through 4 of the message.

  - Call getChunkPacket() with the chunk number and the image data loaded earlier.

  - Send the resulting packet back to the client.

- If the command byte is CMD_IMG_DONE: print a success message and break.

- If any other command byte is received: this is unexpected — send a CMD_IMG_NACK, print a warning, and break out of the sub-loop to abort the transfer and return to the outer loop.

# Part 17 — Client-Side Implementation

Extend the client Python script from Part 12. The client drives the entire transfer — the server just responds to whatever the client requests.

## 17.1  requestTelemetry(cmd_byte)

Refactor the telemetry code from Part 12 into a reusable function:

Function specification Input: cmd_byte (int) — either CMD_TELEM_A or CMD_TELEM_B Output: the raw 32-byte response as a list, or None if it timed out

- Build a 32-byte packet with byte 0 set to the command byte.

- Send it using sendData().

- Call waitForMessage(). If it returns False, print a timeout warning and return None.

- Otherwise read 32 bytes over I2C and return them.

## 17.2  requestImage(filename, save_path)

This is the main function to write. Build it in three stages and test each one before moving on.

### Stage A — Handshake

Goal: negotiate with the server to confirm the file exists and find out how many chunks to expect.

Function specification Inputs: filename (string), save_path (string) Output: True on success, False on any failure

- Build a CMD_IMG_REQUEST packet. Encode the filename as ASCII bytes and place them in bytes 1 through 30.

- Send the packet and wait for a response.

- Check the command byte of the response. If it is CMD_IMG_NACK, print a message and return False.

- If it is CMD_IMG_CONFIRM, extract total_chunks from bytes 2 through 5 as a 4-byte big-endian integer. Extract last_chunk_sz from byte 6.

- Send a CMD_IMG_ACK packet.

Hints To encode a Python string as bytes: call .encode('ascii') on the string, then slice it to a maximum of 30 bytes. You can assign a bytes slice into a bytearray using slice assignment. To decode a 4-byte big-endian integer from a bytes object at a given offset, use the struct module with the '>I' format code.

### Stage B — Bulk Chunk Download

Goal: request every chunk from the server, keep track of which ones arrived, and re-request any that were missed.

- Initialise a dictionary to store received chunks, keyed by chunk number.

- Initialise a list of all expected chunk numbers (0 through total_chunks - 1). This is your ‘missing’ list.

- Enter a loop that continues while the missing list is not empty.

- Inside the loop, create a new empty list called still_missing. Iterate over every chunk number in the missing list:

  - Build and send a CMD_CHUNK_REQ packet with the chunk number in bytes 1–4 as a uint32.

  - Call waitForMessage(). If it times out, add this chunk number to still_missing and move on to the next.

  - Read the response. If the command byte is not CMD_CHUNK_DATA, or if the chunk number in bytes 1–4 does not match what you requested, add this chunk to still_missing and move on.

  - Otherwise, extract the image data bytes. For all chunks except the last, take bytes 5 through 31. For the last chunk, take bytes 5 through 5 + last_chunk_sz. Store the data in the dictionary with the chunk number as the key. Print a progress indicator.

- After the inner loop finishes, set missing = still_missing. If still_missing is not empty, print a message saying how many chunks are being re-requested.

Hint: tracking progress A simple progress line: calculate what percentage of chunks you have received (len(chunks) / total_chunks * 100), then print it using end='\r' to overwrite the same line each time.

### Stage C — Reassembly

Goal: join all the chunk bytes back together in the correct order and save the image.

- Join all the chunks together in order from chunk 0 to chunk (total_chunks - 1). The chunks dictionary makes this straightforward.

- Write the resulting bytes to the save_path.

- Send a CMD_IMG_DONE packet to signal the server that the transfer is complete.

- Return True.

## 17.3  Interactive Loop Update

Add 'i' to the command menu. When pressed:

- Use input() to ask for the filename on the server.

- Ask for a local save path, offering a sensible default.

- Call requestImage() and print the result.

## 17.4  Testing Tips

- Start with a very small image (∿5 KB) to verify the end-to-end protocol works before moving to larger files.

- Open both Thonny windows side by side so you can watch both outputs simultaneously.

- To test re-request: briefly unplug one antenna mid-transfer, then plug it back in. Missing chunks should be automatically re-requested.

- Reduce TIMEOUT_S to 0.5 once everything is working, to speed up transfers significantly.

# Part 18 — Data Types: What You’re Actually Sending

Every number you send over the radio is ultimately a sequence of bytes. This section explains the common numeric types used in embedded systems and how to convert between raw bytes and real Python numbers.

## 18.1  Bits, Bytes and Why Size Matters

A quick reminder 1 bit = a single 0 or 1. 1 byte = 8 bits. Can represent 256 different values (0–255). Every number type below is just a fixed number of bytes with a specific interpretation. The size matters because it sets the range. Pick a type that's big enough for your data but no bigger than necessary — every extra byte costs payload space.

## 18.2  The Common Types

| Type | Size | Range | struct code | Example use |
| --- | --- | --- | --- | --- |
| uint8 | 1 B | 0 to 255 | 'B' | command byte, status flag |
| uint16 | 2 B | 0 to 65,535 | '>H' | small chunk counts, port numbers |
| uint32 | 4 B | 0 to 4,294,967,295 | '>I' | chunk numbers, file sizes |
| int8 | 1 B | −128 to 127 | 'b' | small signed values |
| int16 | 2 B | −32,768 to 32,767 | '>h' | attitude angles ×10 (deg×10) |
| int32 | 4 B | −2,147,483,648 to 2,147,483,647 | '>i' | timestamps, large counters |
| float | 4 B | ∿±3.4 × 10³38 (~7 sig. figs) | '>f' | temperature, voltage, altitude |

Signed vs. unsigned Unsigned types (uint) can only hold zero and positive numbers. They use all 8/16/32 bits for magnitude. Signed types (int) reserve one bit for the sign, so they can hold negative numbers but their positive range is half as large. Rule of thumb: use unsigned for things that can never be negative (chunk numbers, file sizes, counts). Use signed for things that can go negative (angles, temperature differences, velocity).

What about 'long' in Python? In Python 3, int and long are merged — Python integers are arbitrary precision and never overflow. But when you send a number over a radio link as bytes, you have to choose a fixed size. That's what the struct types above are for. 'long' in C/Arduino is 32 bits (same as int32).

## 18.3  Why Raw Bytes Aren’t the Real Number

When the server sends you a temperature of 23.5°C packed as a float, it arrives as 4 bytes:

23.5 as a 32-bit IEEE 754 float: Binary: 0 10000011 01110000000000000000000 Hex: 0x41BC0000 Bytes: [0x41, 0xBC, 0x00, 0x00] (big-endian) If you print those bytes directly you see: [65, 188, 0, 0] That looks nothing like 23.5 — you have to unpack them.

This is why you must always use struct.unpack_from() to reconstruct the real value. The bytes themselves are meaningless without knowing the type they represent.

## 18.4  Reconstructing Telemetry Values

Here is a complete worked example. Suppose the server sent back a CMD_TELEM_A response and you received it as a list of 32 integers:

msg = [0x01, 0x41, 0xBC, 0x00, 0x00, # byte 0 = command, bytes 1-4 = temperature 0x40, 0x6E, 0xB8, 0x52, # bytes 5-8 = voltage 0x43, 0xC8, 0x00, 0x00, # bytes 9-12 = altitude 0x00, 0x00, ...] # bytes 13-31 = zeros

To get the real numbers back you need to:

- Convert the list to a bytes object so struct can read it.

- Call struct.unpack_from() with the right format code and the byte offset of each value.

import struct # Step 1: convert the raw list to bytes raw = bytes(msg) # Step 2: unpack each value at its correct byte offset temperature = struct.unpack_from('>f', raw, 1)[0] # offset 1, 4 bytes -> float voltage = struct.unpack_from('>f', raw, 5)[0] # offset 5, 4 bytes -> float altitude = struct.unpack_from('>f', raw, 9)[0] # offset 9, 4 bytes -> float print(temperature) # -> 23.5 print(voltage) # -> 3.72 print(altitude) # -> 400.0

Why [0] at the end? struct.unpack_from() always returns a tuple, even if you only asked for one value. The [0] picks the first (and only) element out of that tuple. For example: struct.unpack_from('>f', raw, 1) returns (23.5,) and [0] gives you 23.5.

## 18.5  Packing Values to Send

The reverse: writing a real number into a bytearray so you can transmit it:

import struct pkt = bytearray(32) # 32 zero bytes pkt[0] = 0x01 # command byte (simple assignment, it's just 1 byte) # Pack a float into bytes 1-4 struct.pack_into('>f', pkt, 1, 23.5) # format, buffer, offset, value # Pack a uint32 into bytes 5-8 struct.pack_into('>I', pkt, 5, 12345) # Pack a uint16 into bytes 9-10 struct.pack_into('>H', pkt, 9, 999) print(list(pkt[:12])) # -> [1, 65, 188, 0, 0, 0, 0, 48, 57, 3, 231, 0]

## 18.6  Quick Reference: struct Format Codes

| Format | Type | Size | Notes |
| --- | --- | --- | --- |
| '>B' | uint8 | 1 B | Uppercase = unsigned. The '>' prefix means big-endian. |
| '>b' | int8 | 1 B | Lowercase = signed. |
| '>H' | uint16 | 2 B | H for 'Half' (half of a 32-bit word). |
| '>h' | int16 | 2 B |  |
| '>I' | uint32 | 4 B | I for 'Integer'. Used for chunk numbers in this lab. |
| '>i' | int32 | 4 B |  |
| '>f' | float | 4 B | IEEE 754 single-precision. Used for telemetry values. |

⚠️ Always keep your format codes consistent between server and client If the server packs a value as '>f' (float), the client must unpack it as '>f'. If the client tries to unpack it as '>I' (uint32) it will get a meaningless number. The bytes are the same — it's the interpretation that changes. Also keep the endianness consistent. This lab uses big-endian ('>') throughout. Mixing '>' and '<' on different sides is a very common source of subtle bugs where numbers arrive scrambled.

16.831 Radios Lab — Spring 2026 — Written by Alex Choi, James Dingley & Claude